DESIGN CONSIDERATIONS FOR CAMERA-BASED OBJECT COUNTING ON CONVEYOR BELTS

Object-based counting on a conveyor belt, which involves determining the number of objects present over a given period, can be achieved through various approaches. In the present scenario, however, a functioning vision model is already available, which requires image inputs to perform the counting. Therefore, the primary requirement is to establish a reliable method for supplying the model with suitable images, ensuring it operates as intended.

In my research, I have encountered the concept of a Programmable Logic Controller (PLC) [1], which is frequently employed in camera-based object counting on conveyor belts [2], [3]. A PLC is an industrial digital computer designed specifically for the control of manufacturing processes or any activity that demands high reliability, ease of programming, and efficient fault diagnosis [4]. PLCs serve as the central control units for a wide range of automated systems and processes [1], offering multiple inputs and outputs that utilise transistors and other circuitry to simulate switches and relays for equipment control [1]. They are programmed through dedicated software, which interfaces via standard computer ports, proprietary programming languages, and network protocols [1].

With regard to our project, PLCs will form the foundation for coordinating the camera system and conveyor belt, ensuring precise timing, reliable data acquisition, and accurate object counting.

PLC-Camera Integration for Conveyor Belt Object Counting

Connecting a camera to a Programmable Logic Controller (PLC) that controls a conveyor belt is not only feasible but also forms the basis of several advanced industrial automation systems. The methods used to connect the components and manage object tracking vary depending on the required system complexity, speed, and operational precision.

1. Integration Approaches

Two distinct systems in the literature demonstrate the integration of PLCs with cameras for automated object counting:

A. Size-Based Counting System [2]

This system employs a PLC in conjunction with an industrial vision camera to perform real-time counting and classification of objects on a conveyor belt. Key components and their roles include:

- Connection and Communication: A Power Link Bus Controller provides efficient communication and synchronization among the PLC, vision camera, and Human-Machine Interface (HMI).

- Power Supply: Dedicated power modules initialise the system components and supply peripheral devices such as the vision camera.

- Conveyor Control: The PLC manages the conveyor system, which is based on an ACOPOS trak motion platform, ensuring precise motion and positioning for image acquisition.

B. AI-Enhanced Waste Sorting System [5]

This system integrates a PLC-controlled intelligent conveyor with AI-enhanced vision for applications such as waste sorting:

- Connection and Communication: The camera (an ONN 1440p desktop webcam) connects to a personal computer (PC) via a USB interface. The PC executes an AI processing algorithm (YOLOv8) and communicates with the PLC over an Ethernet link. This connection allows data exchange using PLC tag addresses to send control commands and receive system feedback.

- Conveyor Control: An Allen-Bradley PLC receives input signals from the PC and manages the motor control for the conveyor. Communication is facilitated through the Pylogix library, ensuring reliable integration between vision processing and conveyor control.

2. Tracking Object Movement and Preventing Duplicate Counting

Accurate object tracking and the prevention of double counting are achieved through precise motion control and sequential logic implemented on the PLC.

- Real-Time Data (Size-Based Counting): In the size-based counting system, the camera captures each object as it enters a detection zone. The Mapp Vision matching function outputs real-time information, including object position and classification, which is communicated to the PLC. The PLC then applies classification logic to increment the corresponding counts.

- Encoder Tracking (Non-Vision Counting): Some conveyor systems use incremental rotary encoders, which convert belt motion into electrical pulses read by the PLC. Each pulse corresponds to a discrete movement step, enabling the PLC to track object positions and count items based on conveyor motion alone.

- Sequential Logic and Resets (AI-Enhanced Sorting): The AI-enhanced system employs structured workflows managed through PLC ladder logic programmed in RS Logix 5000. This ensures sequential, non-duplicative processing:

  - The conveyor moves the object to the camera position.

  - The camera detects and identifies the object.

  - The PLC triggers a subroutine that loads a preset timer value (T3.PRE) calculated specifically for the detected object. This timer determines the distance the conveyor moves before stopping.

  - The sorting mechanism (stepper motor or actuator) directs the object into the correct bin.

  - All timers are reset, and the cycle repeats for the next object. Latching logic isolates outputs connected to previously processed objects, ensuring accurate sorting and preventing reprocessing.

3. Ensuring Adequate Time for Image Capture

The PLC’s control over the conveyor ensures sufficient time for reliable image acquisition:

- Controlled Halting: In the AI-enhanced sorting system, the conveyor halts momentarily in front of the camera, allowing the camera or the associated PC algorithm to capture and process the image before the object continues along the conveyor.

- Predefined Duration: The PLC program includes predefined motion steps (T1) that advance the conveyor to the detection zone before initiating the camera capture phase. This ensures each object is positioned correctly for imaging and subsequent processing.

Camera Positioning, Setup, and Related Challenges

Guidance on camera positioning is also elaborated within the literature which also discuss several issues and limitations associated with using vision cameras in automated conveyor systems, particularly concerning noise, errors, and object occlusion.

1. Camera Positioning and Setup

The literature provides specific details regarding camera placement in two distinct sorting systems:

A. Size-Based Counting System (B&R Vision Camera)

- The camera is mounted on the conveyor belt to detect object movement.

- It captures images of each object in real time as it enters the detection zone.

B. AI-Enhanced Waste Sorting System (ONN Webcam)

- The system employs an ONN 1440p desktop webcam with autofocus to accommodate variations in object dimensions without requiring manual adjustment.

- The camera is positioned at a right angle to the conveyor belt.

- It is located at the entry point of the conveyor, ensuring that every item is captured immediately for object recognition.

- The system is designed to acquire distinct images as objects temporarily halt for inspection.

2. Issues and Limitations

The studies identify several challenges inherent to real-world vision systems, including hardware constraints, environmental factors, noise, and object overlap (occlusion).

A. Object Overlap and Occlusion

- A major challenge in the AI-enhanced sorting system is handling overlapping objects, which can compromise classification accuracy.

- While the system is effective in sorting objects within disordered streams, developing algorithms capable of accurately differentiating overlapping items remains a critical research focus.

- Future improvements aim to enhance the system’s handling of overlaps by employing advanced image processing algorithms, including improved segmentation and edge detection, to distinguish stacked or adjacent objects.

- Depth-sensing technologies, such as stereo cameras or LiDAR, are being considered to provide spatial information and improve performance in complex scenarios.

B. Environmental and Hardware Limitations

- The B&R Vision Camera system faces constraints related to hardware, environmental conditions, and scalability.

- In the AI-enhanced system, a relatively high class loss was observed, likely due to background interference during paper classification. Introducing a uniform black background on the conveyor belt is suggested to reduce noise and enhance classification accuracy.

C. Data Quality and Image Capture

- Motion blur and insufficient image capture time are mitigated by momentarily halting the conveyor using PLC control. This precise regulation ensures that objects remain stationary in front of the camera, allowing accurate image acquisition before proceeding along the belt.

- The use of an autofocus-enabled camera is critical in maintaining consistent detection accuracy across varying object sizes and positions.

Image Acquisition and Data Transmission to Processing Models

The literature identifies two distinct methods for transmitting images from the camera to the processing model or software for analysis, classification, and counting, depending on the system architecture employed.

1. AI-Enhanced Waste Sorting System (YOLOv8 Model)

In the PLC-controlled intelligent conveyor system with AI-enhanced vision for waste sorting, images are captured by a standard webcam and processed on a separate computer running a deep learning model:

- Camera and Capture: An ONN 1440p desktop webcam is positioned at a right angle to the conveyor belt, ensuring clear and distinct images of individual items.

- Transmission to PC: Captured images are transmitted to a personal computer (PC) via a USB interface.

- Model Processing: The PC executes a part-detection algorithm using the Ultralytics YOLOv8 deep learning package. Image capture is performed using Python’s cv2.VideoCapture(0) command, while real-time inference is carried out with the model (img, stream=True) function, enabling detection and classification of objects within the captured frames.

- Data Flow: Following object recognition, the classification results are sent over an Ethernet connection to the PLC, which subsequently controls the sorting mechanisms according to the processed information.

2. Size-Based Counting System (Mapp Vision Technology)

In the size-based counting system employing B&R Vision technology, image processing is closely integrated with the industrial automation platform (PLC):

- Camera and Capture: A high-performance vision camera captures images of objects in real time as they enter the detection zone.

- Integrated Processing: The system utilises the Mapp Vision matching function within B&R Automation Studio. This function captures monochromatic images and performs size-based template matching against reference markers to classify objects.

- Data Output: Rather than transmitting raw images, the matching function generates processed results, including object position and classification.

- Transmission to PLC: These matching results are output via function blocks to the PLC (X20CP0484), which utilises the data for decision-making and displays relevant information on the Human-Machine Interface (HMI).

CONCLUSION

The integration of cameras with PLC-controlled conveyor systems provides a reliable and flexible framework for automated object counting and classification. Two primary approaches are observed in the literature: one where image processing is performed directly on the industrial automation platform using B&R Automation Studio and Mapp Vision technology, and another where images are captured by a standard webcam and processed on a separate PC running a deep learning model such as YOLOv8.

Both approaches have distinct advantages. Integrated processing on the PLC simplifies system architecture and ensures rapid real-time communication between vision and control components. Conversely, PC-based processing enables the use of advanced AI models and sophisticated image analysis techniques, offering higher flexibility and adaptability to complex tasks.

Key factors affecting system performance include careful camera positioning, environmental control, and precise conveyor motion management via the PLC to prevent duplicate counting and ensure high-quality image capture. Selecting an appropriate architecture depends on the system’s operational requirements, complexity, and desired throughput.

REFERENCES

[1] A. S. Tasu, "Programmable Logic Controller," Department of Physics, Ovidius University, Constanţa, Romania, received Dec. 21, 2004.

[2] N. Thune, A. Kanwade, H. Jadhav, P. Doshi, A. Gherde, S. Jagtap, and S. Kudgunti, "Size Based Counting of Moving Objects on Conveyor Belt using PLC," Marathwada MitraMandal’s College of Engineering, Savitribai Phule Pune University, Pune, Maharashtra, India; B&R Industrial Automation Pvt. Ltd, Pune, Maharashtra, India, available online Aug. 31, 2025. doi: 10.2991/978-94-6463-831-8_45.

[3] S. Redkar, C. Dhargalkar, D. Shirodkkar, V. Kelkar, and P. Volvoikar, "Automated Counting System for Industrial Conveyors," *Int. J. Eng. Res. Technol. (IJERT)*, vol. 9, no. 7, Jul. 2020, doi: 10.17577/IJERTV9IS070211.

[4] M. Jambhulkar, P. Dhanvijay, S. Jambhulkar, H. Bose, P. Bhoyar, and S. Ambade, "Automation of Conveyor Belt in Bricks and Blocks by PLC," *Int. J. Innov. Res. Electr. Electron. Instrum. Control Eng. (IJIREEICE)*, vol. 8, no. 5, May 2020, doi: 10.17148/IJIREEICE.2020.8514.

[5] N. Almtireen, V. Reddy, M. Sutton, A. Nedvidek, C. Karn, M. Ryalat, H. Elmoaqet, and N. Rawashdeh, "PLC-Controlled Intelligent Conveyor System with AI-Enhanced Vision for Efficient Waste Sorting," *Appl. Sci.*, vol. 15, no. 3, Art. no. 1550, Feb. 2025, doi: 10.3390/app15031550.